Submitted by: MUHAMMAD BILAL HAMZA

LINK: https://drive.google.com/file/d/1ssa4WMUWHyhwmPt6O81-xBg6FfBlFRTF/view?usp=drive_link

Objective(s):

The goal of this project is to apply the systems and network administration concepts and skills acquired during lab sessions in a practical scenario.

## Project Requirements:

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| CENTER | S0/2/0 | 192.168.10.2 | 255.255.255.0 | N/A |
|  | S0/2/1 | 192.168.100.2 | 255.255.255.0 | N/A |
|  | S0/3/0 | 192.168.4.3 | 255.255.255.0 | N/A |
|  | S0/3/1 | 192.168.5.3 | 255.255.255.0 | N/A |
|  | S1/0 | 192.168.50.2 | 255.255.255.0 | N/A |
|  | S1/1 | 192.168.60.2 | 255.255.255.0 | N/A |
|  | S1/2 | 192.168.2.3 | 255.255.255.0 | N/A |
|  | S1/3 | 192.168.3.3 | 255.255.255.0 | N/A |
| L1 L2 | G0/0 | 192.168.9.1 | 255.255.255.0 | N/A |
|  | G0/1 | 192.168.10.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 192.168.30.2 | 255.255.255.0 | N/A |
|  | S0/0/1 | 192.168.5.2 | 255.255.255.0 | N/A |
|  | G0/0 | 192.168.7.1 | 255.255.255.0 | N/A |
|  | G0/1 | 192.168.8.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 192.168.30.1 | 255.255.255.0 | N/A |
|  | S0/0/1 | 192.168.4.2 | 255.255.255.0 | N/A |
| SASAS L L1.1 L | NIC | 192.168.9.2 | 255.255.255.0 | 192.168.9.1 |
| L1.2 | NIC | 192.168.9.3 | 255.255.255.0 | 192.168.9.1 |
| L1L L1.3 | NIC | 192.168.9.4 | 255.255.255.0 | 192.168.9.1 |
| DHCP PUNJAB |  | 192.168.10.2 | 255.255.255.0 | 192.168.10.1 |
| DNS SERVER PUNJAB | NIC | 192.168.10.2 | 255.255.255.0 | 192.168.10.1 |
| L2 1.1 | NIC | 192.168.8.2 | 255.255.255.0 | 192.168.8.1 |
| L2 1.2 | NIC | 192.168.8.3 | 255.255.255.0 | 192.168.8.1 |
| FTP SERVER PUNJAB | NIC | 192.168.8.4 | 255.255.255.0 | 192.168.8.1 |
| L2 2.1 | NIC | 192.168.7.2 | 255.255.255.0 | 192.168.7.1 |
| SL2 2.2 | NIC | 192.168.7.3 | 255.255.255.0 | 192.168.7.1 |
| L2 2.3 | NIC | 192.168.7.4 | 255.255.255.0 | 192.168.7.1 |
| AXzxxxXXZ S2 | G/0/0 | 192.168.18.1 | 255.255.255.0 | N/A |
|  | G/0/1 | 192.168.18.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 192.168.50.2 | 255.255.255.0 | N/A |
|  | S0/0/1 | 192.168.3.2 | 255.255.255.0 | N/A |
| DNS SERVER SINDH | NIC | 192.168.19.4 | 255.255.255.0 | 192.168.19.1 |
| S2 2.1 | NIC | 192.168.18.2 | 255.255.255.0 | 192.168.18.1 |
| S2 2.2 | NIC | 192.168.18.3 | 255.255.255.0 | 192.168.18.1 |
| S2 2.3 | NIC | 192.168.18.4 | 255.255.255.0 | 192.168.18.1 |
| DHCP SERVER SINDH | NIC | 192.168.18.5 | 255.255.255.0 | 192.168.18.1 |
| B2 | G/0/0 | 192.168.14.1 | 255.255.255.0 | N/A |
|  | G/0/1 | 192.168.15.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 192.168.40.2 | 255.255.255.0 | N/A |
|  | S/0/0/1 | 192.168.60.1 | 255.255.255.0 | N/A |
| B1 1.1 | NIC | 192.168.13.2 | 255.255.255.0 | 192.168.13.1 |
| B1 1.2 | NIC | 192.168.13.3 | 255.255.255.0 | 192.168.13.1 |
| FTP BAL | NIC | 192.168.13.4 | 255.255.255.0 | 192.168.13.1 |
| B1 2.1 | NIC | 192.168.12.2 | 255.255.255.0 | 192.168.12.1 |
| B1 2.2 | NIC | 192.168.12.3 | 255.255.255.0 | 192.168.12.1 |
| B1 2.3 | NIC | 192.168.12.4 | 255.255.255.0 | 192.168.12.1 |
| B2 2.1 | NIC | 192.168.14.4 | 255.255.255.0 | 192.168.14.1 |
| DHCP-BAL | NIC | 192.168.14.3 | 255.255.255.0 | 192.168.14.1 |
| B2 2.2 | NIC | 192.168.14.2 | 255.255.255.0 | 192.168.14.1 |
| B2 2.3 | NIC | 192.168.14.5 | 255.255.255.0 | 192.168.14.1 |
| B2 2.4 | NIC | 192.168.14.6 | 255.255.255.0 | 192.168.14.1 |
| DNS SERVER BAL | NIC | 192.168.15.2 | 255.255.255.0 | 192.168.15.1 |

You are tasked with designing, installing, and configuring a network infrastructure for an organization of your choice using Cisco Packet Tracer. The network must meet the following specifications:

- The organization should have a head office and at least two branch offices.

HEAD OFFICE IS PUNJAB

BRANCH 1 IS BALOUCHISTAN

BRANCH 2 IS SINDH

- Establish WAN connectivity between the offices using routers.

- Configure at least two VLANs at one of the office locations to segment the internal network.

- Set up and configure VoIP (Voice over IP) communication between all offices.

On CENTER router

For branches and head offices switches

On each branch router :

- Deploy and configure a DNS server for name resolution.

DNS HAS BEEN DONE IN THE NETWORK

- Use DHCP to automatically assign IP addresses across the network.

Before assigning DHCP

After assigning DHCP

DHCP IS ALSO BUILT ACROSS THE NETWORK

- Implement NAT (Network Address Translation) to enable internet access for internal hosts.

- Install and configure a web server within the network.

For headoffice , branches all web servers have been separately allocated

- Apply Access Control Lists (ACLs) to regulate and secure traffic flow between the head office and the branch offices.

ACL IS ENABLED BETWEEN HALF OF SINDH AND BALOUCHISTAN, you can see picture to

Help you understand

- Set up and configure a Cisco firewall to safeguard the network from external threats.

This project is intended to reinforce your understanding of enterprise-level network design and security through hands-on implementation.

| Done | Requirement | Status |
| --- | --- | --- |
| Yes | The organization should have a head office and at least two branch offices | Complete |
| Yes | Establish WAN connectivity between the offices using routers | Complete |
| Yes | Configure at least two VLANs at one of the office locations to segment the internal network | Complete |
| Yes | Set up and configure VoIP (Voice over IP) communication between all offices | Complete |
| Yes | Deploy and configure a DNS server for name resolution | Complete |
| Yes | Use DHCP to automatically assign IP addresses across the network | Complete |
| Yes | Implement NAT (Network Address Translation) to enable internet access for internal hosts | Complete |
| Yes | Install and configure a web server within the network | Complete |
| Yes | Apply Access Control Lists (ACLs) to regulate and secure traffic flow between the head office and the branch offices | Complete |
| Yes | Set up and configure a Cisco firewall to safeguard the network from external threats | Complete |

Project Assessment Rubric – System and Network Administration

|  |  |  |  |  |  | Conformance Level |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conformance Legend |  |  |  |  |  | Excellent | Good | Fair | Poor | Extremely Poor |
| Conformance Level %age |  |  |  |  |  | (80-100)% | (60-80)% | (40-60%) | (20-40)% | (0-20)% |
| Rating |  |  |  |  |  | 4.5 – 5.0 | 3.5 – 4.0 | 2.5 – 3.0 | 1.5 – 2.0 | 0.0 – 1.0 |
| Sr. No. | Criteria |  | Description | Skill | Marks Weight age |  |  |  |  |  |
| 1. |  | Design | This portion reflects the design portion of the project including diagrams timelines and outlines,. | Logical and physical Topologies | 6 | Design is complex, includes flow chart, and timeline. Goals and written description of project are present. Schematics exist and are readable. Design has been updated to represent final project, and all versions are included. | Design is mostly complete, some updates, but not current. | Basic design, Not updated. | Little or no design present. | No design |
| 2. |  | Hardware | The hardware part of the project includes building the network. | Network Hardware knowledge | 4 | Network hardware correctly implemented all components communicate; network is consistent with design | Project is built, all hardware components are implemented network communicates but numerous problems are present does not match design. | All required network hardware has been implemented, however not working. | Only one or two hardware components have been implemented, network does not communicate. | No Hardware used. |

| 3. | Software | This is the software component of this project. Students will install server and client software and make appropriate network connections | Knowledge of NOS, and Simulator | 6 | All software is installed and working correctly, all networking tests are passed. | Most software is installed, and installed components are working correctly, most networking test are passed. | All software is installed however not correctly working. | Not all software is installed on all machines. | No software installed at all. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. | Testing and Documentation | Students run appropriate network tests and document all problems encountered and corrections made. | Problem Solving | 4 | Great All tests were completed successfully and full documentation exists on any problems encountered, and solutions applied. | All test were completed, successfully limited documentation on problems and corrections. | Some testing was done, problems documented but corrections not implemented or documented. | Testing was not done appropriately. | No testing done at all. |